NetLinx Module Interface Specification

for a

RGB Spectrum DualView

Seamless Switching & Side-By-Side Display

TABLE OF CONTENTS

Introduction	3

Overview	3

Implementation	3

Command Interface	5

Extended Command Interface	7

String Feedback	14

Device Notes	18

Programming Notes	18

LIST OF TABLES

Table 1 – Send Command Definitions	6

Table 2 - Extended Send Command Definitions	13

Table 3 - String Feedback Definitions	18

Revision History

| ###### Date | Initials | Comments |
| --- | --- | --- |
| 07-07-2003 | CGA | Initial Release |
| 07-18-2003 | CGA | Fixed copy/paste errors in HUE= and SATURATION= commands. |

## Introduction

This is a reference manual to describe the interface provided between an AMX NetLinx system and a RGB Spectrum DualView video windowing system. The DualView supports an RS-232 serial protocol. The interface was tested using version 1.08 of the firmware. The required communication settings are a baud rate of 9600, 8 data bits, 1 stop bit, no parity, and handshaking off. The cable for this device is FG#10-752-10. The wiring for this cable is as follows:

AMX 4-pin phoenix – DB9 Male

1 GND – 5 GND

2 RX – 2 RX

3 TX – 3 TX

## Overview

The COMM module translates between the standard interface described below and the DualView serial protocol. It parses the buffer for responses from the windowing system, sends strings to control the windowing system, and receives commands from the UI module or telnet sessions.

A User Interface (UI) module is also provided. This module uses the standard interface described below and parses the string responses for feedback.

The following diagram gives a graphical view of the interface between the interface code and the NetLinx module.

DualView

NetLinx

COMM Module

NetLinx

UI Module

Virtual

Device

## Implementation

To interface to the AMX RGB_Spectrum_DualView_Com.axs  module, the programmer must perform the following steps:

- Define the device ID for the DualView that will be controlled.

- Define the virtual device ID that the DualView COMM module will use to communicate with the main program and User Interface. NetLinx virtual devices start with device number 33001.

- If a touch panel interface is desired, a touch panel file RGB Spectrum DualView-Platinum 640x480,Rev1.TPD and module ( RGB_Spectrum_DualView_UI.axs) have been created for testing.

- The NetLinx DualView module must be included in the program with a DEFINE_MODULE command. This command starts execution of the module and passes in the following key information: the device ID of the DualView to be controlled, and the virtual device ID for communicating to the main program.

An example of how to do this is shown below.

DEFINE_DEVICE

dvDevice		= 5001:1:0	// The DualView connected to the NetLinx on 1st RS-232 port

dvTouchPanel 	= 128:1:0	// The touch panel used for output

vdvDevice       	= 33001:1:0	// The virtual device use for communication between the

// Comm module interface and User_Interface (UI) module interface

DEFINE_VARIABLE

//Define arrays of button channels used on your own touch panel

integer nTP_BUTTONS[]={1,2,3,4,5,6,7}

DEFINE_START	// Place define_module calls to the very end of the define_start section.

// Comm module

DEFINE_MODULE ‘Device_Comm’ Comm1(dvDevice, vdvDevice)

// Touch panel module

DEFINE_MODULE 'User_Interface' TouchPanelA(vdvDevice, dvTouchPanel, nTP_BUTTONS)

Upon initialization the AMX Comm module will communicate with the DualView and information will be exchanged.

## Command Interface

The UI module controls the DualView via command events (NetLinx command send_command) sent to the COMM module. The commands supported by the COMM module are listed below.

| Command | Description |
| --- | --- |
| ASPECT=<state> | Sets the current aspect ratio. <state> : 1 = Normal 5 = Stretched 6 = Cropped ASPECT=1 |
| BRIGHTNESS=<value>[:<input>] | Adjust the brightness level for the specified input. If no input is specified, brightness is set for all inputs. <value> = 0..100 = API range -500..500 = Device specific range <input> = 1-2 = selected input BRIGHTNESS=60 BRIGHTNESS=100:2 |
| BRIGHTNESS?[<input>] | Query for the current brightness setting. If no input is specified, then all input channels are queried. Input : 1-2 = selected input BRIGHTNESS?2 |
| CONTRAST=<value>[:<input>] | Adjust the contrast level of the specified input. If no input is specified, contrast is set for all inputs. <value> = 0-100 = API range 0-200 = Device specific range <input> = 1-2 = selected input CONTRAST=30 CONTRAST=50:2 |
| CONTRAST?[<input>] | Query for the current contrast setting for the specified input. If no input is specified, all input channels are queried. <input> : 1-2 = selected input CONTRAST?2 |
| DEBUG=<state> | Set the state of the telnet debugging messages. <state>: 0 = off 1 = on DEBUG=1 |
| DEBUG? | Query the state of the telnet debugging messages. DEBUG? |
| DEVICE_SCALE=<value> | Sets the number scale used by commands and strings. <state>: 0 = Use standard API range (1..100) 1 = Use hardware specific range (device specific) DEVICE_SCALE=0 |
| DEVICE_SCALE? | Queries for the number scale. DEVICE_SCALE? |
| PASSTHRU=<string> | Allows user the capability of sending commands directly to whatever unit is attached without processing by the NetLinx module. User must be aware of the protocol implemented by the unit to use this command. This gives the user access to features, which may not be directly supported by the module. <string> : string to send to unit PASSTHRU=THIS IS A COMMAND PASSTHRU=RESET |
| SHARPNESS=<value>[:<input>] | Adjust the sharpness level for the specified input. If no input is specified, then sharpness is set on all inputs. <value> : 0-100 = API range 0-3 = Device specific range <input> : 1-2 = selected input SHARPNESS=52 SHARPNESS=25:2 |
| SHARPNESS?[<input>] | Query for the current sharpness setting of the specified input. If no input is specified, then all input channels are queried. <input> : 1-2 = selected input SHARPNESS?2 |
| VERSION? | Query for the current version number of the NetLinx module. VERSION? |
| VIDEOMODE? | Query for the current video format of the video on input source 1. Note: This command is only valid for Composite, Component, or S-Video input types. This command will return ERRORM= for all RGB input types. VIDEOMODE? |

Table 1 – Send Command Definitions

## Extended Command Interface

This windowing system requires some “extended” commands that go beyond the standard command interface described above.  These extended commands are listed below.

| Command | Description |
| --- | --- |
| BAUD? | Query the current baud rate. BAUD? |
| DOUBLE_BUFFER=<input>:<state> | Set the double buffering feature for a specific input. <input> : 1-2 = selected input <state> : OFF ON DOUBLE_BUFFER=2:ON |
| DOUBLE_BUFFER?<input> | Query the current double buffering setting for a specific input. <input> : 1-2 = selected input DOUBLE_BUFFER?2 |
| FADE=<input> | Fades the currently active input source to the specified input source, or setting. <input> : RGB1 = Input 1 RGB RGB2 = Input 2 RGB COMPOSITE = Input 1 Composite COMPONENT = Input 1 Component SVIDEO = Input 1 S-Video DVN = Normal DualView mode DVS = Stretch DualView mode DVC = Cropped DualView mode BLACK = Fade to black FADE=RGB2 |
| FADE_TIME=<time> | Sets the duration of the FADE= command action. The time is in seconds. <time> : 1.5-2.0 = seconds FADE_TIME=1.5 |
| FADE_TIME? | Query the current time setting for the FADE= command. FADE_TIME? |
| FIRMWARE? | Query the firmware version of the DualView. FIRMWARE? |
| FREEZE=<state>[:<input>] | Freeze or unfreeze the specified input. If no input is specified, then freeze is applied to all inputs. <state> : 0 = unfreeze 1 = freeze T = toggle (only valid when input specified) <input> : 1-2 = selected input FREEZE=1 FREEZE=0:2 |
| FREEZE?[<input>] | Query for the current freeze state for the specified input. If no input is specified, then all input channels are queried. <input> : 1-2 = selected input FREEZE?2 |
| FULL_SCREEN=<input> | Select a single input, or no input, to be displayed full screen. Note: Selecting 0 will result in a black screen with no inputs visible. <input> : 0 = No input 1 = Input 1 2 = Input 2 FULL_SCREEN=1 |
| FULL_SCREEN? | Query for the current input being displayed full screen. FULL_SCREEN? |
| GAMMA=<value>[:<input>] | Adjust the gamma value for the specified input source. If no input source is selected, then gamma is set for all input sources. <value> : 0-100 = API range 0.5-2.0 = Device specific range <input> : 0 = side-by-side gamma setting 1-2 = selected input GAMMA=52 GAMMA=33:1 |
| GAMMA?[<input>] | Query for the current gamma setting for the specified input source. If no input is selecte,d then gamma is queries for all inputs. <input> : 0 = side-by-side gamma setting 1-2 = selected input GAMMA?1 |
| GLOBAL_SAVE | Force an update and explicit save of the system’s NVRAM. GLOBAL_SAVE |
| HUE=<value>[:<input>] | Adjust the hue level for the specified input. If no input is specified, then hue is set on all inputs. Note: This command is only valid for input channels set for video, not RGB. <value> : 0..100 = API range -180..180 = Device specific range <input> : 1-2 = selected input HUE=52 HUE=25:2 |
| HUE?[<input>] | Query for the current hue setting of the specified input. If no input is specified, then all input channels are queried. Note: This command is only valid for input channels set to video, not RGB. <input> : 1-2 = selected input HUE?2 |
| INPUT_MODE=<input>:<state> | Set the input mode for the specified input. <input> : 1-2 = Specific input channel <state> : AUTO = Engages autosync circuitry LOCK = Disengages autosync circuitry DEBUG = Provides status INPUT_MODE=1:AUTO |
| INPUT_MODE?<input> | Query the mode for the specified input. <input> : 0 = All input channels 1-2 = selected input channel INPUT_MODE?2 |
| INPUT_NAME=<input>:<name> | Set the name for a specified input with a locked signal. Note: The name may not contain any spaces, but it may contain underscores. <input> : 1-2 = specific input source <name> : string of up to 17 alphanumeric characters INPUT_NAME=2:DVD_Player |
| INPUT_NAME?<input> | Query for the assigned name of a specified input source. <input> : 1-2 = specific input source INPUT_NAME?2 |
| INPUT_SAVE=<input>:<place> | Save the specified input to the selected entry in the Input List. Note: This command is for RGB input sources only. <input> : 1-2 = specified input source <place> : 1-50 = place in Input List INPUT_SAVE=2:50 |
| INPUT_TIMING=<input>:<mode>[:<direction>] | Enters input interactive mode to visually adjust timing parameters of the specified input. Modes LR and UL require the optional direction parameter. Note: This command is not for use with input channels set to video, only RGB. <input> : 1-2 = specified input channel <mode> : E = enable input timing Q = disable input timing LR = adjust lower right corner UL = adjust upper left corner <direction> : + = move corner up - = move corner down < = move corner left > = move corner right INPUT_TIMING=1:E INPUT_TIMING=1:LR:+ INPUT_TIMING=1:UL:< INPUT_TIMING=1:Q |
| INPUT_TYPE=<input>:<type> | Select the video type for the specified input. <input> : 1-2 = selected input <type> : COMPOSITE S-VIDEO COMPONENT RGB INPUT_TYPE=1:S-VIDEO |
| INPUT_TYPE?<input> | Query the type for the selected input. <input> : 1-2 = selected input INPUT_TYPE?1 |
| INPUT_WINDOW=<input>:<x>:<y>:<width>:<height> | Set the input window (input source rectangle) for a specific input. Note: Video inputs are limited to 700x480. RGB inputs are limited to 1280x1024. <input> : 1-2 = specific input source <x> : X-coordinate for top left corner of input window <y> : Y-coordinate for top left corner of input window <width> : pixel width of input window <height>: pixel height of input window INPUT_WINDOW=1:0:0:400:300 |
| INPUT_WINDOW?<input> | Query for the current dimensions of the specified input window (input source rectangle). <input> : 1-2 = specific input source INPUT_WINDOW?1 |
| MAP=<left>:<right> | Sets up the input/window mapping assignment. <left> : 1-2 = input source to be on left side of display <right> : 1-2 = input source to be on right side of display MAP=1:2 MAP=2:1 |
| MOVE=<input>:<direction> | Move a specified input’s window around the output display. Note: The currently active window must be disabled before another window can be enabled. <input> : 1-2 = selected input <direction> : E = enable moving functionality S = disable moving functionality > = move right < = move left + = move up - = move down MOVE=2:< |
| OPTIMIZE_OUTPUT=<state> | Improves horizontal resolution. Note: This command is most useful for CRT output devices with a resolution less than 1280x1024. If output device is a discrete sampled device, this may cause vertical banding. <state> : 0 = Off 1 = On OPTIMIZE_OUTPUT=1 |
| OUTPUT_NAME=<name> | Set the name for the current output source. Note: The name may not contain any spaces, but it may contain underscores. <name> : string of up to 17 alphanumeric characters OUPUT_NAME=1:Monitor |
| OUTPUT_NAME? | Query the name of the current output source. OUTPUT_NAME? |
| OUTPUT_SAVE=<place> | Save the current output source settings into the Host List. <place> : 1-10 = place in the Host List OUTPUT_SAVE=4 |
| OUTPUT_TIMING=<mode>[:<direction>] | Enters output interactive mode to visually adjust timing parameters of the output device. Modes LR and UL require the optional direction parameter. <mode> : E = enable input timing S = disable input timing LR = adjust lower right corner UL = adjust upper left corner <direction> : + = move corner up - = move corner down < = move corner left > = move corner right OUTPUT_TIMING=E OUTPUT_TIMING=LR:+ OUTPUT_TIMING=UL:< OUTPUT_TIMING=S |
| OUTPUT_WINDOW=<input>:<x>:<y>:<width>:<height> | Set the output window (input’s destination rectangle) for a specific input source. <input> : 1-2 = specific input source <x> : X-coordinate for top left corner of output window <y> : Y-coordinate for top left corner of output window <width> : pixel width of output window <height>: pixel height of output window OUTPUT_WINDOW=2:0:0:1024:768 |
| OUTPUT_WINDOW?<input> | Query the current dimensions of the output window (input’s destination rectangle) for a specific input source. <input> : 1-2 = input source OUTPUT_WINDOW?1 |
| OVER_SCAN=<input>:<state> | Set the on/off state of the overscan functionality for a specific input. <input> : 1-2 = selected input <state> : OFF ON OVER_SCAN=1:ON |
| OVER_SCAN?<input> | Query the current setting of the overscan functionality for a specific input. <input> : 1-2 = selected input OVER_SCAN?2 |
| PANTILT=<input>:<value> | Move the input’s view up, down, left, and right. <input> : 1..2 = selected input <value> : 1 = begin tilt down - = begin pan right - = begin tilt up 7 = begin pan left S = disable panning and tilting E = enable panning and tilting PANTILT=1:5 PANTILT=2:S |
| PIP=<foreground>:<background> [:<x>:<y>:<width>:<height>] | Sets the Picture-In-Picture mode on. The optional x, y, width, and height parameters allow sizing of the foreground window. <foreground> : 1-2 = input <background> : 1-2 = input <x> : -2400..2400 = x-coordinate of the upper left corner <y> : -2400..2400 = y-coordinate of the upper left corner <width> : 0-1200 = width of the foreground window <height> : 0-1200 = height of the foreground window PIP=1:2 PIP=1:2:0:0:100:100 |
| RESET_INPUT_WINDOW=<input> | Resets the specified input window (input source rectangle) to its default values. <input> : 1-2 = specific input source RESET_INPUT_WINDOW=1 |
| RESTORE_DEFAULTS | Restores all user settings to their factory default values. Note: This command make take a few moments to complete. RESTORE_DEFAULTS |
| SATURATION=<value>[:<input>] | Adjust the saturation level for the specified input. If no input is specified, then saturation is set on all inputs. Note: This command is only valid for input channels set to video, not RGB. <value> : 0-100 = API range 0-200 = Device specific range <input> : 1-2 = selected input SATURATION=52 SATURATION=25:2 |
| SATURATION?[<input>] | Query for the current saturation setting of the specified input. If no input is specified, then all input channels are queried. Note: This command is only valid for input channels set to video, not RGB. <input> : 1-2 = selected input SATURATION?2 |
| SIZE=<input>:<direction> | Allows the specified input’s window to be resized. <input> : 1-2 = selected input <direction> : + = make larger - = make smaller S = finished resizing E = enable resizing SIZE=2:+ |
| TEST_PATTERN=<state> | Set the on/off state of the test pattern. <state> : 0 = Off 1 = On TEST_PATTERN=0 |
| TEST_PATTERN? | Query the state of the test pattern option. TEST_PATTERN? |
| VIDEO=<input>:<state> | Turn the video window for the specified input on or off. <input> : 0 = all inputs 1-2 = selected input <state> : 0 = off - = on VIDEO=2:1 |
| VIDEO?[<input>] | Query the on/off state of the specified input’s window. If no input is specified, then all inputs are queried. <input> : 1-2 = selected input VIDEO?2 |
| ZOOM=<input>:<val> | Zoom input in (near/tele) or out (far/wide). <input> : 1..2 = input to affect <val> : + = begin zoom in (tele) - = begin zoom out (wide) S = disable zoom functionality E = enable zoom functionality ZOOM=1:+ |

Table 2 - Extended Send Command Definitions

## String Feedback

The NetLinx COMM module provides feedback to the User Interface module for DualView changes via string events. The strings supported are listed below.

| String | Description |
| --- | --- |
| ASPECT=<state> | Feedback on the current aspect ratio. <state> : 1 = Normal 5 = Stretched 6 = Cropped ASPECT=1 |
| BAUD=<value> | Feedback on the current baud rate at which communication between the NetLinx and the DualView is taking place. <value> : 1200 2400 9600 19200 38400 115200 BAUD=9600 |
| BRIGHTNESS=<value>:<input> | Feedback on the current brightness setting of the specified input. <value> : 0-100 = API percent range -500..500 = device specific range <input> : 1-2 = selected input BRIGHTNESS=50:2 |
| CONTRAST=<value>:<input> | Feedback on the current contrast setting of the specified input. <value> : 0-100 = API percent range 0-200 = device specific range <input> : 1-2 = selected input CONTRAST=50:2 |
| DEBUG=<state> | Feedback on the on/off state of the telnet debugging messages. <state> : 0 = Off 1 = On DEBUG=1 |
| DEVICE_SCALE=<state> | Feedback on the on/off state of device specific ranges used with commands and strings. <state> : 0 = Off 1 = On DEVICE_SCALE=0 |
| DOUBLE_BUFFER=<input>:<state> | Feedback on the state of the double buffering feature for a specific input. <input> : 1-2 = selected input <state> : OFF ON DOUBLE_BUFFER=2:ON |
| ERRORM=<msg> | Feedback on an error that occurred within the Comm module or the physical device. <msg> : string ERRORM=Invalid command. |
| FADE=<input> | Feedback on the last fade setting. <input> : RGB1 = Input 1 RGB RGB2 = Input 2 RGB COMPOSITE = Input 1 Composite COMPONENT = Input 1 Component SVIDEO = Input 1 S-Video DVN = Normal DualView mode DVS = Stretch DualView mode DVC = Cropped DualView mode BLACK = Fade to black FADE=RGB2 |
| FADE_TIME=<time> | Feedback on the duration of the FADE= command action. The time is in seconds. <time> : 1.5-2.0 = seconds FADE_TIME=1.5 |
| FIRMWARE=<version> | Returns the firmware version of the physical device. <version> : string FIRMWARE=1.1 |
| FREEZE=<state>:<input> | Feedback on the on/off freeze state of a specific input’s window. <state> : 0 = Off 1 = On <input> : 1-2 = selected input FREEZE=1:2 |
| FULL_SCREEN=<input> | Feedback on which input is being displayed full screen. <input> : 0 = No input 1 = Input 1 2 = Input 2 FULL_SCREEN=1 |
| GAMMA=<value>:<input> | Reports the current gamma setting for the specified input. <value> : 0-100 = API percent range 0.5-2.0 = device specific range <input> : 0 = side-by-side diplay setting 1-2 = selected input GAMMA=50:1 |
| HUE=<input>:<value> | Reports the current hue setting for a specific input source. <input> : 1-2 = input source <value> : 0-100 = API percent range -180..180 = device specific range HUE=2:50 |
| INPUT_MODE=<input>:<state> | Feedback on the current input mode of the specified input. <input> : 1-2 = selected input <state> : AUTO = autosync circuitry is engaged LOCK = autosync circuitry is disengaged DEBUG = providing status INPUT_MODE=2:AUTO |
| INPUT_NAME=<input>:<name> | Feedback on the name for a specified input with a locked signal. <input> : 1-2 = specific input source <name> : string of up to 17 alphanumeric characters INPUT_NAME=1:DVD_Player |
| INPUT_TYPE=<input>:<type> | Feedback on the video type selected for the specified input. <input> : 1-2 = selected input <type> : COMPOSITE S-VIDEO COMPONENT RGB INPUT_TYPE=2:RGB |
| INPUT_WINDOW=<input>:<x>:<y>:<width>:<height> | Feedback on the input window (input source rectangle) for the specified input. <input> : 1-2 = specified input source <x> : X-coordinate for top left corner of input window <y> : Y-coordinate for top left corner of input window <width> : pixel width of input window <height>: pixel height of input window INPUT_WINDOW=1:0:0:400:300 |
| MAP=<left>:<right> | Sets up the input/window mapping assignment. <left> : 1-2 = input source to be on left side of display <right> : 1-2 = input source to be on right side of display MAP=1:2 MAP=2:1 |
| OPTIMIZE_OUTPUT=<state> | Feedback on the state of the horizontal resolution improvement feature. <state> : 0 = Off 1 = On OPTIMIZE_OUTPUT=1 |
| OUTPUT_NAME=<name> | Feedback on the name set for the current output source. <name> : string of up to 17 alphanumeric characters OUPUT_NAME=Monitor |
| OUTPUT_WINDOW=<input>:<x>:<y>:<width>:<height> | Feedback on the output window (input’s destination rectangle) for a specific input source. <input> : 1-2 = specific input source <x> : X-coordinate for top left corner of output window <y> : Y-coordinate for top left corner of output window <width> : pixel width of output window <height>: pixel height of output window OUTPUT_WINDOW=2:0:0:1024:768 |
| OVER_SCAN=<input>:<state> | Feedback on the on/off state of the overscan functionality for the specified input. <input> : 1-2 = selected input <state> : OFF ON OVER_SCAN=1:ON |
| PIP=<foreground>:<background> [:<x>:<y>:<width>:<height>] | Feedback on the Picture-In-Picture mode setting. The optional x, y, width, and height parameters return the sizing of the foreground window. <foreground> : 1-2 = input <background> : 1-2 = input <x> : -2400..2400 = x-coordinate of the upper left corner <y> : -2400..2400 = y-coordinate of the upper left corner <width> : 0-1200 = width of the foreground window <height> : 0-1200 = height of the foreground window PIP=1:2 PIP=1:2:0:0:100:100 |
| SATURATION=<input>:<value> | Returns the saturation value as a percent for the specified input source. <input> : 1-2 = input source <value> : 0-100 = API percent range 0-200 = device specific range SATURATION=2:50 |
| SHARPNESS=<value>:<input> | Feedback on the current sharpness setting of the specified input. <value> : 0-100 = API percent range 0-3 = device specific range <input> : 1-2 = selected input SHARPNESS=50:2 |
| TEST_PATTERN=<state> | Feedback on the current test pattern setting. <state> : 0 = Off 1 = On TEST_PATTERN=1 |
| VERSION=<value> | Reports the current version number of the NetLinx module. <value> : current version number in xx.yy format VERSION=1.06 |
| VIDEO=<input>:<state> | Feedback on the on/off state of the input’s video window. <input> : 1-2 = selected input <state> : 0 = Off 1 = On VIDEO=1:1 |
| VIDEOMODE=<mode> | Feedback on the video format on input source 1. <mode> : 1 = NTSC 2 = PAL VIDEOMODE=2 |

Table 3 - String Feedback Definitions

## Device Notes

## Programming Notes