| ### Release Notes |  |
| --- | --- |
| ### Product | #### QVHD, WATCHDOG 100/200 |
| VERSION: | 1.2 |
| PART # / CSUM: |  |
| COMPATIBILITY: | V1.0 & V1.1 |
| NEW FEATURES: | Add new timing 77: 2048x1152.60p |
| CHANGES: | The following changes are included in this release. |
|  | Replaced VESA_1280x960_120RB with VESA_1280x960_100RB to allow locking to this resolution. |
|  | Added QFX_2048x1080,60p and QFX_2048x1080,24p timing support |
|  | Changed Hostlist: Put old QVX 2048 60p into SONYDCI slot |
|  | Renamed QVX2048 24p to SonyDCI 24p |
| BUG FIXES: | The following bug fixes are included in this release. |
|  | Resolved input locking problems with cable and satellite set-top boxes. |
|  | Resolved 2048 input timing support problems |
| ANOMALIES: | The following discrepancies are known in this release |
|  | The system administrator password does not change via WCP. |
|  | The Analog HD15 connector does not support 720x480,60I inputs. To use this format, it is necessary to use the 16x4 Routing Switcher, BNC connectors and configured as a COMPONENT signal rather than an RGB signal. |
|  | Custom Timing entry via WCP has occasional failures. The work-around is to re-apply the timing one or more times , until the numbers stick. |
| RELEASE DATE | February 30, 2009 |